Name:Framed Aluminium Tube SMT Laser Stainless Steel Stencil

Material: Stainless Steel with Aluminium Tube

Finish:Laser Cut

Stencil thickness capability:0.1mm,0.12mm, 0.13mm, 0.15mm, 0.18mm, 0.2mm

| Frame Size(CM) | Tube(CM) | Capable Stainless Steel Size（CM） | Price(USD) |
| --- | --- | --- | --- |
| 30*40 | 30*40*2 | 14*24 | 80 |
| 37*47 | 37*47*2 | 18*28 | 90 |
| 42*45 | 42*45*2 | 23*33 | 90 |
| 55*65 | 55*65*3 | 33*43 | 100 |
| 73.6*73.6 | 73.6*73.6*3 | 50*50 | 200 |
| 80*45 | 80*45*3 | 60*25 | 100 |
| 70*40 | 70*40*3 | 50*20 | 100 |
| 120*40 | 120*40*3 | 100*20 | 150 |
| 140*50 | 140*50*3 | 120*30 | 250 |
| 140*40 | 140*40*3 | 120*20 | 200 |
| 100*40 | 100*40*3 | 80*20 | 130 |
| 40*60 | 40*60*2.5 | 20*40 | 100 |

Other specific size is capable.

Note

Firstly,We will make a stencil according to paste layer except the following issue we can’t open them:

1,Standard gold fingers in paste layer;

2,SMT key in paste layer;

3,RF antenna in paste layer;

Secondly,We will generate paste layer according to soldermask,copper,drill layers if there is no paster layer in your gerber file,following are our generate rules:

1,VIAs uncut;

2,SMT key,gold fingers,RF antenna uncut;

3,The shield or soldermask on traces in solder layer(just to increase power on purpose),please give me your cut way;

4,If there is no copper in this soldermask area correspondingly,we will uncut;

5,No pad reduction in our stencil,we will cut same size according to your paste layer;

6,Please let me know where and which way you would like us to add Fiducials in your stencils,thanks in advance.

7,We will cut other areas except 1),2),3),4),5)6)

Finally, and Importantly:If there is only paste layer you provided,we will cut this stencil according to this file and no further enquiry,please note this.